This file contains information on the NET/ROM support and Finger support

which I did not have time to get merged into USERMAN.DOC before Dayton.

The NET/ROM information was provided by Dan Frank W9NK, author of the

NET/ROM support for NET.EXE... and the Finger docs were provided by

Mike Horne KA7AXD, author of the finger server.

73 - Bdale, N3EUA

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

The NET/ROM support for the KA9Q package serves three purposes:

1) Existing NET/ROM networks may be used to send IP traffic.

2) NET may be used as a NET/ROM packet switch.

3) NET may be used to communicate with NET/ROM nodes, and its

mailbox facility can accept connects over the NET/ROM network.

.Nh 2 "Setting up the NET/ROM Interface"

.LP

No physical interface is completely dedicated to net/rom, which is as

it should be.  You attach all your AX.25 interfaces, of whatever sort.

Then you attach the net/rom pseudo-interface ("attach netrom").  Then

you identify to the net/rom software those interfaces you want to allow

it to use, with the "netrom interface" command.  The format of this

command is:

.LP

netrom interface ax0 #ipnode 192

.LP

The first argument is the name of the previously attached interface you

want to use.  The second argument is the alias of your node, to be used in

your routing broadcasts.  The alias is never used for anything else (as

you will see!).  The last number is the net/rom quality figure.  This is

used in computing the route qualities; it represents the contribution of

this interface to the overall computation.  For a 1200 baud half-duplex

connection, 192 is the right number.

.LP

You need a netrom interface command for every interface you're going

to use with net/rom.

.Nh 2 "Tracing on the NET/ROM Interface"

.LP

If you want to trace your NET/ROM datagrams, don't try turning

on trace mode for the "netrom" interface.  Nothing will break, but

nothing will happen.  You should trace the individual AX.25 interfaces

instead.

.Nh 2 "Routing Broadcasts"

.LP

Once you have set up your interfaces, you need to set some timers.

There are two:  the nodes broadcast interval timer, and the obsolescence

timer.  These are set in seconds, like the smtp timer.  You should usually

set them to an hour.  You can set them to something different, if you want.

If your local net/rom nodes broadcast every hour, but you want to do so

every ten minutes, you can say:

.LP

netrom nodetimer 600

netrom obsotimer 3600

.LP

Every time the obsotimer kicks, the obsolescence counts for all non-permanent

entries are decremented by one.  When the count for an entry falls below

five, it is no longer broadcast.  When it falls to 0, it is removed.  The

count is initialized at 6.  These will eventually be settable parameters;

you can adjust them now by changing the initializers for the variables

in the source file.

.LP

When you first come on the air, you can send out nodes broadcasts to

tell the local nodes that you are available.  Use the command:

.LP

netrom bcnodes ax0

.LP

where ax0 is the interface on which you want to send the broadcast.  Do

this for every interface on which you want to do this.

.LP

By default, the NET/ROM code does not broadcast the contents of your

routing table.  This is as it should be, since usually we just want to

be the endpoints of communications rather than relaying NET/ROM traffic.

If you want to be a switch station, include the command:

.LP

netrom verbose yes

.LP

in your autoexec.

.LP

Sometimes you can hear broadcasts from nodes that can't hear you.  If

your routing table gets filled with these unusable routes, your node will

grind to a halt.  The solution to this is node broadcast filtering, via

the netrom nodefilter command.  There is a filter list, which contains

a list of callsigns and interfaces.  Then there is a filter mode, which

indicates what to do with the list.

.LP

If the filter mode is "none", no filtering is done.  If it is "accept",

then only broadcasts from the indicated stations on the indicated

interfaces are accepted.  If it is "reject", then all broadcasts

except those from the listed stations on the listed interfaces are

accepted.

.LP

Because the net/rom code cannot at this time recognize unusable

routes and try alternates, I strongly recommend use of the filter

command to restrict broadcast acceptance to those nodes which you

know you can reach.

.Nh 2 "The NET/ROM Routing Table"

.LP

The next net/rom commands are those used for maintaining

the routing table.  They fall under the "netrom route" subcommand.

"netrom add" adds a permanent entry to the routing table.  Its format

is:

.LP

netrom route add #foo w9foo ax0 192 w9rly

.LP

This command adds an entry for w9foo, whose alias is #foo, route

quality 192, via w9rly on interface ax0.  Let's talk about what this

means.  w9foo is the *destination* node, the one to whom you want

the packets routed by the net/rom network.  w9rly is your *neighbor*,

the net/rom node to which you pass the packet to be forwarded.  Since

w9rly may appear on more than one interface (the callsign may be used

by more than one net/rom node on different bands), we specify that

we are to use ax0 to send the packet.

.LP

With net/rom, like IP, we don't know exactly what route a packet

will take to its destination.  We only know the name of a neighbor

which has indicated a willingness to forward that packet (of course,

the neighbor may be the destination itself, but that's unlikely in

our application).  Net/rom sends the packet to the neighbor, with a

network header specifying our callsign and that of the ultimate

destination (in this case w9foo).

.LP

We can use the netrom route add command to establish a digipeater

path to the neighbor.  For example:

.LP

netrom route add #foo w9foo ax0 192 w9rly wd9igi

.LP

This will cause us to use wd9igi as a digipeater in establishing our

connection to the net/rom node w9rly.

.LP

To drop the route to w9foo, you would type

.LP

netrom route drop w9foo w9rly ax0

.LP

To see the contents of your routing table, you may type

.LP

netrom route

.LP

and to see the routing entries for an individual station you can type

.LP

netrom route info <callsign>

.LP

You may not use an alias as an argument to the netrom route info command.

.LP

I can not stress enough that "route add" and "netrom route add" are two

different commands, with different purposes.  In general, you only need a

"netrom route add" if you need to add a route to a net/rom node via a

digipeater path.  If you find yourself using this command, ask yourself,

"Why am I doing this?"  Many people do not understand that net/rom does

automatic routing (well, sort of :-)).

.Nh 2 "The Importance of the Routing Table"

.LP

The NET/ROM routing table is analogous to the IP routing table:  if

there is nothing in it, your NET/ROM traffic will not go out.  You must

either manually enter a list of routes (perhaps via your autoexec.net)

or wait to receive routing broadcasts from your neighbors before your

NET/ROM traffic will leave your station.

.LP

If you go to send packets via NET/ROM and nothing happens, even if

you have trace mode on, make sure that the destination node is in your

NET/ROM routing table.  If sending IP traffic, double check the ARP table

for an appropriate NET/ROM ARP entry for the destination node (see below

for more information on the use of the ARP table).  The ARP table is not

used for NET/ROM transport routing.

.Nh 2 "Interfacing with NET/ROMs Using Your Serial Port"

.LP

What if you have a net/rom node or nodes, and you'd like to attach

them to your computer via their serial interfaces, and use net as a

packet switch?  It's very easy:  you have to attach those interfaces,

using the "attach asy" command, but specifying type "nrs" instead of

"slip" or "kiss".  "nrs" is the net/rom serial framing protocol, which

is like KISS, but uses different framing characters and has an 8-bit

checksum.

.LP

When you attach an nrs interface, it can be used for passing IP

datagrams or AX.25 frames over serial lines or modems.  To use it

for net/rom, you have to identify it to the netrom code just like

any other interface, with the "netrom interface" command.

.Nh 2 "The Time to Live Initializer"

.LP

The "netrom ttl" command allows setting of the time-to-live

initializer for NET/ROM datagrams.  I recommend a value of 16

for most networks.  Use more if you expect to go more than 16 hops.

The default is 64.

.LP

The purpose of the ttl initializer is to prevent a packet from

getting caught forever in routing loops.  Every router who handles

the packet decrements the ttl field of the network datagram before

sending it on, and when it reaches 0 it is discarded.

.Nh 2 "Using NET/ROM Support for IP"

.LP

Now you know all the commands, but how do we actually use net/rom

for IP communications?  This takes two steps:

.LP

Step one:  update the routing table.  In all likelihood, you will

use net/rom to gateway two IP subnets.  So, you'll probably want to

identify a station on each end as a gateway.  Let's say we're on the

Milwaukee subnet, and we want to talk to someone in Madison.  If

we're not the gateway, we just have a routing table entry like this:

.LP

route add [44.92.0.0]/24 ax0 wg9ate-pc.ampr

.LP

This specifies that wg9ate should get all packets for the 44.92.0.x

subnet via interface ax0.

.LP

Wg9ate has this routing table entry:

.LP

route add [44.92.0.0]/24 netrom w9mad-pc.ampr

.LP

(presuming that w9mad is the Madison gateway).  Now, when the IP layer

at wg9ate gets datagrams for Madison, it knows that they have to go via

net/rom to w9mad.  Notice that we don't specify a "real" interface,

like ax1 or nr0, in the route entry.  The net/rom network layer will

pick the right interface based on its net/rom routing tables.

.LP

We're not done yet, though.  w9mad-pc.ampr is not an ax.25

callsign.  The net/rom send routine called by the IP layer needs

to map from the IP address to an ax.25 address.  It does this via

a manually added arp entry:

.LP

arp add w9mad-pc.ampr netrom w9mad

.LP

[We kind of fudged by using the arp table for this purpose, since

there is no way to do automatic address resolution for net/rom,

and arp messages are never sent or received for net/rom nodes.

However, the arp table does contain precisely what we have here:

mappings from IP addresses to callsigns, and it saved a lot of

code to do it this way.]

.LP

Notice also that no digipeaters are ever specified in the arp entry

for a net/rom node.  Also, the callsign to which we are mapping

is the final destination of the packet, not the non-destination

neighbor.  That neighbor will be picked based on the net/rom

routing tables.

.LP

So, as a summary, let's look at what happens to a packet that

reaches the IP layer on wg9ate, destined for Madison.  The IP

routing code looks the destination IP address up in the table,

and discovers that it should go via net/rom to w9mad-pc.ampr.

So, it passes the packet to the net/rom send routine.  That

routine uses the arp table to translate w9mad-pc's IP address

to the callsign "w9mad".  Then it passes the packet to the

net/rom routing code.  That code checks to see if the destination

callsign (w9mad) is the same as that of any of its assigned

net/rom interfaces.  Since it isn't, it puts a network layer

header (a.k.a. net/rom level 3 header) on it, and looks for

w9mad in its routing tables.  Presumably, it finds an appropriate

neighbor for the packet, and sends in out via ax.25.  The net/rom

network does the job of actually getting the packet to its

destination.

.LP

At w9mad, the packet's protocol ID causes it to be sent to

the same net/rom routing code that handled the outgoing packet

from wg9ate (running on a different computer, of course).  Now

the destination callsign matches, so the net/rom network layer

header is stripped off, and packet is passed up to the IP layer.

(Net/rom network headers don't have a protocol ID byte, so

we just hope for the best.  If a net/rom node addresses a

net/rom transport layer packet to us, it is likely to be dropped

by IP for any of a number of reasons.)

.Nh 2 "The NET/ROM Transport Layer"

.LP

NET/ROM transport is the protocol used by NET/ROM node to

communicate end-to-end.  When a user attaches to a NET/ROM

via AX.25, and asks for a connect to a node in the NODES list,

his local NET/ROM tries to open a transport connection to

the destination node over the NET/ROM network.  NET/ROM transport

packets are carried in NET/ROM network datagrams, just like

IP datagrams.

.LP

You shouldn't use NET/ROM transport when connecting to other

TCP/IP stations.  TCP is a much better protocol than NET/ROM

transport, and makes better use of available bandwidth.  Also,

BM and SMTP are more convenient to use than a TCP/IP station's

mailbox facility.  However, for communicating with AX.25 users

via the NET/ROM network, the transport facilities in NET will

work better (and more easily) than the traditional method of

connecting to your local node via AX.25.

.Nh 2 "Connecting via NET/ROM Transport"

.LP

To connect to the node whose alias is FOO and whose callsign is

W9FOO, you can issue either of the following two commands:

.LP

netrom connect foo

.LP

netrom connect w9foo

.LP

If foo:w9foo is in your NET/ROM routing table, your station will

transmit a connect request to the appropriate neighbor used to

reach w9foo.

.LP

NET/ROM transport sessions are very much like those for AX.25.

You can use the disconnect, reset, kick, upload, and record

commands, and the session command to switch sessions.

.Nh 2 "Displaying the Status of NET/ROM Connections"

.LP

The command

.LP

netrom status

.LP

is used to display the status of all NET/ROM connections, which will

include those used in keyboard sessions as well as ones attached to

the mailbox.  For more detailed information on a session, you can

use the address of the NET/ROM control block:

.LP

netrom status <&nrcb>

.LP

where <&nrcb> is the hex address given in the short form of the command

or in the "session" display.

.Nh 2 "NET/ROM Transport Parameters"

.LP

The NET/ROM transport parameters may be set with the various

NET/ROM subcommands.  Their meanings are listed below:

.LP

acktime:	This is the ack delay timer, similary to ax25 t2.

The default is 3000 ms.

choketime:	The time to wait before breaking a send choke condition.

Choke is the term for NET/ROM flow control.

irtt:		The initial round trip time guess, used for timer setting.

qlimit:		The maximum length of the receive queue for chat sessions.

This is similar to ax25 window.

retries:	Maximum retries on connect, disconnect, and data frames.

window:		Maximum sliding window size, negotiated down at connect

time.

.Nh 2 "The Mailbox"

.LP

The AX.25 mailbox also accepts NET/ROM connections.  The "mbox on"

and "mbox off" commands control whether the mailbox is turned on for

NET/ROM as well as AX.25, and the "mbox" command displays current

mailbox connects of both types.

.LP

Many people have observed that the AX.25 mailbox requires the user

to enter a carriage return to bring up the banner and prompt.  This is

because of certain defects of that protocol when it is used as the

link layer for several different higher level protocols, and is

unavoidable.  (So stop asking, OK? :-))  The NET/ROM mailbox does

not require the carriage return, and will be activated as soon as

the incoming connection is completed.

.Nh 2 "Where to go for More Information"

.LP

The paper "Transmission of IP Datagrams over NET/ROM Networks"

appeared in the Seventh ARRL Networking Conference papers, available

from the ARRL.  In it, I describe the more technical details of how

the NET/ROM network support works.

.LP

If you want to learn about NET/ROM, talk your local NET/ROM or TheNET

operator out of his or her manual.  If you want to learn more, read

the source code.  That's about it for sources, since the NET/ROM

protocols originated in a commercial product.

.Nh 2 "About the Code"

.LP

There has been a great deal of controversy about TheNET, a no-charge

NET/ROM clone for TNCs.  This is not the place to discuss the truth

of the charges leveled by Software 2000 against its authors, but that

situation requires me to make the following statement:

.LP

The NET/ROM transport support in NET.EXE was not taken in any way,

shape or form from NET/ROM (whose source I have never seen) or from

TheNET.  The protocol code is based on protocol 6 from Tanenbaum's

excellent book, Computer Networks, as a moderately careful reading

of both should show.  The source code is freely distributed, so

the curious reader should have the opportunity to check this assertion

if he or she so desires.

.LP

The smoothed round trip time calculation, which is not done in

"real" NET/ROMs (and should be, by the way -- they'd work a whole

lot better) is adapted from that used by KA9Q in the TCP protocol

in NET.  The dicey business of adapting it to a sliding windows

protocol with selective retransmission was done by me, all alone,

after my cries for help on the tcp-group mailing list went unanswered :-).

.LP

I have taken the precaution of copyrighting the NET/ROM code in

NET.  It may be freely distributed for non-commercial purposes, in

whole or in part, and may be used in other software packages such

as BBS systems if so desired, so long as the copyright notice is

not removed from the source files, and the program in which it is

used displays "NET/ROM code copyright 1989 by Dan Frank, W9NK"

when it starts up.

.LP

Any person who wishes to distribute the code, or anything based

on the code, for commercial purposes will find me very reasonable,

but rather insistent about being compensated for the hours I've

spent working on it.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

.Nh 1 "The Finger Server"

.Nh 2 "Finger Support Release Docs"

.LP

UPDATED: August 17, 1988 - KA7AXD

.LP

This ARC file has everything you need to add finger to the alpha.arc sources

that Phil has placed on flash.bellcore.com .  Simply replace the .C and .H

files, as well as the MAKEFILE, from the alpha.arc archive with those in

this archive and away you go!

.LP

I have merged in Dan Frank's (W9NK) NET/ROM enhancements into the code as

well, which included some fixes to the NET/ROM code and the AX.25 Mailbox

feature.  Read the file "readme.dan" for his comments.

.LP

I've made some corrections to fingserv.c and fingcli.c that Gerard (PA0GRI)

has suggested.  These corrections help eliminate the potential for causing

DoubleDos to lock up when using finger.  Thanks, Gerard!  A few other

small changes have been made to provide for more robust code.

.LP

If you have any questions or comments, please direct them to me at one of

the addresses below.

.LP

Enjoy!

.LP

Mike, ka7axd, mhorne@honda.fdi.tek.com